Product Main Parameter/Function

| Camera | Description |
| --- | --- |
| Chip | CV1821 |
| Sensor | SC3335/GC4653/SC401AI/OV4689/SC4336 |
| Resolution | 2304x1296P、2560x1440P |
| Diagonal angle of view | 106° |
| Lens focal length | 4mm,6mm |
| Night vision function | Infrared night vision ,Color night vision |
| AV | Description |
| Video coding protocol | H.264/H.265 |
| Video code rate | 80Kbps~1Mbps(self-adaption) |
| Audio compression method | G.711A/AAC |
| Function | Description |
| 3D digital noise reduction | Support |
| Wide dynamic | Support |
| Backlight compensation | Support |
| White balance | Support |
| Human shape detection | Support |
| Motion detection | Support |
| Voice detection | Support |
| Cloud broadcasting | Support |
| Aural-visual alarm | Support |
| Network | Description |
| Network Protocol | L2TP, IPv4, IGMP, ICMP, ARP, TCP, UDP, DHCP,RTP,RTSP, RTCP,UPnP, DNS,PPPoE,DDNS,NTP, FTP,HTTP, HTTPS,SIP, 802.1x |
| Interface | Description |
| Audio input and output | Audio wiring, 1 wires input, 2 wires output |
| Power supply | 12V⎓1A,PoE 48V⎓0.3A |
| Button | Reset button |
| Network interface | RJ45 10M/100M |
| Storage | Description |
| Local storage | Micro SD, support a maximum of 128GB |
| Operation | Description |
| Working environment | -20℃ to +60℃ |

CV1821+SC3335/GC4653/SC401AI/OV4689/SC4336

Wiring diagram

| Interface | Description |
| --- | --- |
| J4 | 1:12V_IN 2:USB_DP 3:USB_DM 4:GND |
| J8 | 1:VCC3.3 2:VCC1.8 3:VCC1.2 4:SEN_PWDN 5:CAM_RST 6:CAM_I2C_SCL 7:CAM_I2C_SDA 8:GND 9:CAM_MCLK 10:GND 11:CAM_RX_0N 12:CAM_RX_0P 13:GND 14:CAM_RX_1N 15:CAM_RX_1P 16:GND 17:CAM_RX_2N 18:CAM_RX_2P 19:GND 20:CAM_RX_3N 21:CAM_RX_3P 22:GND 23:CAM_RX_4N 24:CAM_RX_4P 25:GND 26:IR_CUT_P 27:IR_CUT_N 28:IRC_PWR |
| J7 | 1:MIC_IN 2:GND |
| J3 | 1:GPIO1 2:GPIO2 3:GPIO3 4:GPIO4 5:GPIO5 6:GND |
| J6 | 1:SPK- 2:SPK+ |
| J9 | 1:RCA_OUT 2:GND |
| J2 | 1:IRLED 2:WLED 3:CDS |
| J5 | 1:12V_IN 2:GND 3:ETH_RX- 4:ETH_RX+ 5:ETH_TX- 6:ETH_TX+ 7:PHY_LED 8:KEY |